# FPGA to look at

### Foc alg basics

- Foc

- id/iq currents

- Why use clark/park transforms

- What do pi controllers regulate

- How inverse park turns voltage commands -> phase commands

- How SVPWM/pwm turns voltage commands -> gate sig

Links to look at for ^

- Field-Oriented Control - MATLAB & Simulink

- Clarke and Park Transforms - MATLAB & Simulink

- Space Vector Modulation (SVM) for Motor Control

ADC phase currents -> Clarke -> Park -> id/iq PI loops -> inverse Park -> SVPWM/PWM.

### Fixed point math

- Floating point -> fixed oint math

- Q format

  - Esp. Q1.15 & Q2.30

- Per unit normalization

- Multiply bit growth

- Roudnigns vs truncation

- Saturation vs overflow

- PI controller anti windup

- Inntegers needing extra bits

Links to look at for ^

- Fixed-Point Designer - MATLAB

- Get Started with Fixed-Point Designer

- Iterative Fixed-Point Conversion in Simulink

0.5 pu becomes 16384 in Q1.15 and Q1.15 * Q1.15 makes a Q2.30 intermediate that has to be shifted + rounded + saturated

### MATLAB/Simulink

- How to sim. PMSM/BLDC motor-control loops

- How to log id/iq/voltage cmds/speed/angle/pwm duty

- Simulink blocks -> HDL using hdl coder

- What blocks are hdl friendly

Links

- Motor Control Blockset - MATLAB

- FOC of PMSM Using FPGA-Based Motor Control Development Kit - MATLAB & Simulink Example

- Basic Control Algorithm Partitioned into FPGA and ARM - MATLAB & Simulink

MATLAB/Simulink to test alg first

HDL Coder/Vivado after fixed-point stuff works

### Arty Z7 / Zynq

- ARM processing sys

- AXI lite registers

- Pins available on arduinoheader

- Safe voltage levels

LInks

- Arty Z7 Reference Manual

- Arty Z7 - Digilent Reference

- Zynq 7000 SoC Technical Reference Manual (UG585) - 1.15 English

### AXI-Lite + register control

- Memory mapped i/o

- AXI lite

- How arm writes duty cycle / enable / mode / PI gains

- FPGA reporting fault status / hall states / current samples / speed

Link

- AXI General Purpose IO

### Current sensing and XADC

- Z7  XADC input limits

- XADC sample rates

- Single ended vs differential analog inputs

- Does current sensor voltage range fit z7

- Pwm center sampling

  - Can it be generated from fpga logic

Links

- Arty Z7 Reference Manual

- This guide serves as a technical reference describing the Xilinx 7 series FPGAs and Zynq-7000 AP SoCs XADC, a dual 12-bit, 1 MSPS analog-to-digital converter with on-chip sensors. - UG480

### Shibal Vivado

- XDC constraints

- Syntehticisze + implement + gnereating bit streams

Links

- Getting Started with Vivado - Digilent Reference

- Installing Vivado, Xilinx SDK, and Digilent Board Files

# Notes for stuff I looked at

Pi controllers

- Regulates error between target iq (from throttle)  and measured iq (from sensors)

SVPWM

- Trnasltates target voltage vectors to physical duty cycles sent to gate driver

Fixed point dsp

- Q format

  - Scales limits for amps and volts to range of -1.0 -> 1.0 pu.

    - Pu = per unit

  - Q1.15 format represents fractions as 16 bit ints

- Bit growth

  - Multiplying 2 16 bit nums creates 32 bit num so you have to physically shift + rount + truncate it back into 16 bits

- Saturation + anti windup

  - If calc exceeds the max limit we don’t want it to overflow (bc it might wrap around to a neg num and reverse motor)

  - Saturation logic makes sures it doesn’t exceed the max val

  - Anti windup logic in the pi controller to stop the errors if the voltage is alr maxed out

# MATLAB

# Notes for script

Q1.15

- Represents fractional nums. Using 16 bit int.

  - 1 sign bit + 15  fractional bits

- Eg.

  - Converting decimal 0.5 to Q1.15 you multiply it by 2^15 which makes 0.5 become an actual fixed point integer 16384.

    - Q_SCALE, Q_MAX, Q_MIN

      - These prevent integer overflow

PI current sim

- Most important part of FOC alg

  - PI controllers force motor’s actual current to match target current

- Anti windup

  - Logic to stop accumulating integral error if target voltage hits max limit (0.95 pu)

Clark & park transforms

- 3 phase adc (ia,ib,ic)  -> dc reference frames (id,iq)

- Clarke

  - Abc -> alpha, beta

- Park

  - Rotates stationary clarke frame using motor rotor angle

  - Alpha beta -> d, q

PWM center alingedsample timing

- Used to switch MOSFETS

  - Visualize timer counting up & down making a triangle wave

- Important because adc has to be triggered to read motor current at the exact center of pwm cycle

  - No mosfets switching = lowest noise

# script

Simulation

- Q1.15 fixed point math simulating floating point math

clear; close all; clc;

% base values + Q format

base_current_A = 100;

base_voltage_V = 96;

f_pwm_Hz = 20000;

dt = 1 / f_pwm_Hz;

sim_time_s = 0.04;

t = 0:dt:sim_time_s;

Q_FRAC = 15;

Q_SCALE = 2^Q_FRAC;

Q_MAX = int32(2^15 - 1);

Q_MIN = int32(-2^15);

fprintf("Q1.%d scale = %d\n", Q_FRAC, Q_SCALE);

fprintf("0.5 pu encoded = %d\n", q_encode(0.5, Q_SCALE, Q_MIN, Q_MAX));

fprintf("0.25 pu encoded = %d\n\n", q_encode(0.25, Q_SCALE, Q_MIN, Q_MAX));

%% floating-point and fixed-point PI current loop demo

%% makes target current and initializes variables for both controlerls

iq_ref_pu = zeros(size(t));

iq_ref_pu(t >= 0.004) = 0.55;

iq_ref_pu(t >= 0.025) = -0.25;

kp_pu = 0.32;

ki_pu_per_sample = 0.018;

plant_tau_s = 0.0028;

plant_alpha = dt / plant_tau_s;

iq_float = zeros(size(t));

vq_float = zeros(size(t));

integ_float = 0;

iq_fixed = zeros(size(t));

vq_fixed = zeros(size(t));

integ_q15 = int32(0);

kp_q15 = q_encode(kp_pu, Q_SCALE, Q_MIN, Q_MAX);

ki_q15 = q_encode(ki_pu_per_sample, Q_SCALE, Q_MIN, Q_MAX);

%% main sim loop

for k = 2:numel(t)

% Floating-point PI.

err_float = iq_ref_pu(k) - iq_float(k - 1); % calculates error

[vq_float(k), integ_float] = pi_float_step(err_float, kp_pu, ki_pu_per_sample, integ_float);  % c

iq_float(k) = iq_float(k - 1) + plant_alpha * (vq_float(k) - iq_float(k - 1)); % comment

% Fixed-point PI. Inputs quantized to Q1.15.

ref_q15 = q_encode(iq_ref_pu(k), Q_SCALE, Q_MIN, Q_MAX);

meas_q15 = q_encode(iq_fixed(k - 1), Q_SCALE, Q_MIN, Q_MAX);

err_q15 = sat16(int32(ref_q15) - int32(meas_q15), Q_MIN, Q_MAX);

[vq_q15, integ_q15] = pi_fixed_step(err_q15, kp_q15, ki_q15, integ_q15, Q_FRAC, Q_MIN, Q_MAX);

vq_fixed(k) = q_decode(vq_q15, Q_SCALE);

% Plant remains floating point here but the controller output is fixed point.

iq_fixed(k) = iq_fixed(k - 1) + plant_alpha * (vq_fixed(k) - iq_fixed(k - 1));

end

iq_error_pu = iq_float - iq_fixed;

fprintf("Max float-vs-fixed current error = %.6f pu = %.3f A\n\n", ...

max(abs(iq_error_pu)), max(abs(iq_error_pu)) * base_current_A);

figure("Name", "Fixed-point PI current loop");

subplot(3, 1, 1);

plot(t, iq_ref_pu, "k--", t, iq_float, "b", t, iq_fixed, "r:");

grid on;

ylabel("i_q (pu)");

legend("ref", "float", "fixed", "Location", "best");

title("Current-loop PI: floating point vs Q1.15 fixed point");

subplot(3, 1, 2);

plot(t, vq_float, "b", t, vq_fixed, "r:");

grid on;

ylabel("v_q cmd (pu)");

legend("float", "fixed", "Location", "best");

subplot(3, 1, 3);

plot(t, iq_error_pu * base_current_A);

grid on;

xlabel("time (s)");

ylabel("error (A)");

%% Clarke/Park transform check

theta = linspace(0, 2*pi, 800);

phase_amp_pu = 0.45;

ia = phase_amp_pu * cos(theta);

ib = phase_amp_pu * cos(theta - 2*pi/3);

ic = phase_amp_pu * cos(theta + 2*pi/3);

[i_alpha, i_beta] = clarke_float(ia, ib);

[id_float, iq_float_tr] = park_float(i_alpha, i_beta, theta);

[id_fixed, iq_fixed_tr] = park_q15_demo(ia, ib, theta, Q_SCALE, Q_FRAC, Q_MIN, Q_MAX);

figure("Name", "Clarke/Park transform check");

subplot(2, 1, 1);

plot(theta, id_float, "b", theta, id_fixed, "r:");

grid on;

ylabel("i_d (pu)");

legend("float", "fixed", "Location", "best");

title("Park transform output for balanced 3-phase current");

subplot(2, 1, 2);

plot(theta, iq_float_tr, "b", theta, iq_fixed_tr, "r:");

grid on;

xlabel("electrical angle (rad)");

ylabel("i_q (pu)");

legend("float", "fixed", "Location", "best");

%% PWM center sample strobe timing

pwm_period_counts = 5000;

counter = 0:(2*pwm_period_counts - 1);

updown_counter = [0:(pwm_period_counts - 1), pwm_period_counts:-1:1];

sample_strobe = updown_counter == pwm_period_counts;

duty_a = 0.62;

compare_a = round(duty_a * pwm_period_counts);

pwm_a = updown_counter < compare_a;

figure("Name", "PWM-center sample timing");

stairs(counter, updown_counter, "Color", [0.2 0.2 0.2]);

hold on;

stairs(counter, pwm_a * pwm_period_counts, "b");

stem(counter(sample_strobe), updown_counter(sample_strobe), "r", "filled");

grid on;

xlabel("clock count");

ylabel("counter / PWM");

legend("center-aligned counter", "PWM A", "sample strobe", "Location", "best");

title("Current sample strobe at PWM center");

%% helper functions

function y = q_encode(x, scale, q_min, q_max)

y = int32(round(x .* scale));

y = min(max(y, q_min), q_max);

end

function x = q_decode(y, scale)

x = double(y) ./ scale;

end

function y = sat16(x, q_min, q_max)

y = int32(min(max(int32(x), q_min), q_max));

end

function y = q_mul_q15(a_q15, b_q15, q_frac, q_min, q_max)

raw = int64(a_q15) * int64(b_q15);

if raw >= 0

raw = raw + int64(2^(q_frac - 1));

else

raw = raw - int64(2^(q_frac - 1));

end

shifted = int64(floor(double(raw) / 2^q_frac));

y = sat16(shifted, q_min, q_max);

end

function [out, integ_next] = pi_float_step(err, kp, ki, integ)

integ_candidate = integ + ki * err;

unsat = kp * err + integ_candidate;

out = min(max(unsat, -0.95), 0.95);

pushing_high = (out >= 0.95) && (err > 0);

pushing_low = (out <= -0.95) && (err < 0);

if pushing_high || pushing_low

integ_next = integ;

else

integ_next = integ_candidate;

end

end

function [out_q15, integ_next_q15] = pi_fixed_step(err_q15, kp_q15, ki_q15, integ_q15, q_frac, q_min, q_max)

p_q15 = q_mul_q15(err_q15, kp_q15, q_frac, q_min, q_max);

i_step_q15 = q_mul_q15(err_q15, ki_q15, q_frac, q_min, q_max);

integ_candidate = sat16(int32(integ_q15) + int32(i_step_q15), q_min, q_max);

unsat = sat16(int32(p_q15) + int32(integ_candidate), q_min, q_max);

out_limit = int32(round(0.95 * 2^q_frac));

out_q15 = int32(min(max(unsat, -out_limit), out_limit));

pushing_high = (out_q15 >= out_limit) && (err_q15 > 0);

pushing_low = (out_q15 <= -out_limit) && (err_q15 < 0);

if pushing_high || pushing_low

integ_next_q15 = integ_q15;

else

integ_next_q15 = integ_candidate;

end

end

function [alpha, beta] = clarke_float(ia, ib)

alpha = ia;

beta = (ia + 2 .* ib) ./ sqrt(3);

end

function [id, iq] = park_float(alpha, beta, theta)

id = alpha .* cos(theta) + beta .* sin(theta);

iq = -alpha .* sin(theta) + beta .* cos(theta);

end

function [id, iq] = park_q15_demo(ia, ib, theta, scale, q_frac, q_min, q_max)

one_over_sqrt3_q15 = q_encode(1 / sqrt(3), scale, q_min, q_max);

n = numel(theta);

id = zeros(1, n);

iq = zeros(1, n);

for idx = 1:n

ia_q15 = q_encode(ia(idx), scale, q_min, q_max);

ib_q15 = q_encode(ib(idx), scale, q_min, q_max);

alpha_q15 = ia_q15;

beta_sum_q15 = sat16(int32(ia_q15) + 2 * int32(ib_q15), q_min, q_max);

beta_q15 = q_mul_q15(beta_sum_q15, one_over_sqrt3_q15, q_frac, q_min, q_max);

cos_q15 = q_encode(cos(theta(idx)), scale, q_min, q_max);

sin_q15 = q_encode(sin(theta(idx)), scale, q_min, q_max);

alpha_cos = q_mul_q15(alpha_q15, cos_q15, q_frac, q_min, q_max);

beta_sin = q_mul_q15(beta_q15, sin_q15, q_frac, q_min, q_max);

alpha_sin = q_mul_q15(alpha_q15, sin_q15, q_frac, q_min, q_max);

beta_cos = q_mul_q15(beta_q15, cos_q15, q_frac, q_min, q_max);

id_q15 = sat16(int32(alpha_cos) + int32(beta_sin), q_min, q_max);

iq_q15 = sat16(-int32(alpha_sin) + int32(beta_cos), q_min, q_max);

id(idx) = q_decode(id_q15, scale);

iq(idx) = q_decode(iq_q15, scale);

end

end

# internet

Gits to look at

TexasInstruments/motor-control-sdk · GitHub

merkelmarrow/pynq-esc · GitHub

- Look at pwm_kill block and axi regs for iq_ref from arm -> fpga

GitHub - WangXuan95/FPGA-FOC: An FPGA-based Field Oriented Control (FOC) for driving BLDC/PMSM motor. 基于FPGA的FOC控制器，用于驱动BLDC/PMSM电机。

- Cordic algs for park transform

- Prolly can use modules and put them into axi ip core in vivad

# Arty Z7 Zync 7000 SoC

# Research Notes

sssssssssssssssssssssssssssssssssssssssurg pcb / fpga / foc notes

q’s

- pcb / gate driver side has to make the high/low gate drive + deadtime happen

- need to know where exactly inversion + deadtime happen

- nOC should prob go straight to fpga for fast shutdown? need to check

- current sensor outputs need to fit arty z7-10 xadc range

- only 2 current sensors rn

- so are we reconstructing phase C?

- phase C would be Ic = -Ia - Ib

- confirm this is actually what we want and not accidental

current schematic stuff

- U1 U2 = TMCS1126A1AQDVGRQ1 current sensors

- U3 U4 U5 = UCC21222D isolated dual gate drivers

- Q1-Q6 = BSC110N15NS5 150 V nmos

- D1-D6 = NRVUA140VT3G fast rectifier diodes

- diodes are prob bootstrap / gate driver related

connectors / terminals

- J0 = 5 pin samtec connector near hall stuff

- J0 labels = HALL A HALL B HALL C 12V GND1

- J1 + J4 = 8 pin samtec headers

- J2 = 6 pin samtec header, pinA0-pinA5

- J3 = 10 pin header

- MP1-MP7 = M6 high current pcb power terminals

labels

- Vm

- 12V

- 5V

- 3V3

- GND1

- GND2

- GND1 for Logic, GND2 for HV

- CURR REF A

- CURR SENSE A

- nOC A

- CURR REF B

- CURR SENSE B

- nOC B

- PHASE A+

- PHASE A-

- PHASE B+

- PHASE B-

- PHASE C

- GATE HI A/B/C

- GATE LO A/B/C

- PWM HI A/B/C

- PWM LO A/B/C

- DISABLE

random notes alrwritten in schematic

- Overcurrent = 100A, VOC = 0.8

- Current Sensing x2

- Gate Drivers + MOSFETS x3

- LOW PASSES FOR PWM OUTPUT FROM MCU

- Configurable around deadtime resistors

- CHANGE FOOTPRINTS HERE, RECALCULATE VALUES

- ADD LEDs, TEST POINTS?

- RIGHT FOOTPRINT BUT SHOULD ALL BE NORMAL HEADERS?

- GET RID OF PWM LO INPUTS AND USE INVERTER + MOVE AROUND INPUTS AFTER

- CHANGE NON NUMBER DESIGNATORS

layout things

- 4 copper layers looks like

- top copper about 2 oz

- bottom copper about 2 oz

- inner layers about 1 oz

- core FR-4

- rules still look kind of default

rules

- clearance 8 mil

- board outline clearance 7.874 mil

- default routing width 8 mil preferred

- via width 23.622 mil

- via hole 11.811 mil

- component clearance 5 mil

- supply nets include 3V3 5V GND1 GND2 12V Vm

- supply-rule voltages were 0.000 lol so prob not set

before sending out pcb

- set HV / high current net classes

- real clearances

- real trace widths

- check copper pours

- check thermal assumptions

- run DRC after rules are actually set

- do not trust 8 mil default rules for 96 V / phase current / switch nodes / isolation

parts to double check

- current sensor = TMCS1126A1AQDVGRQ1

- older notes say TMCS1126BDx-Q1 and 20 mV/A sensitivity

- so exact suffix + scaling needs checking

- gate driver = UCC21222D

- mosfet = BSC110N15NS5

- confirm suffixes / packages / footprints

- replace placeholder resistor + capacitor values

- repeated 1k resistors cannot just stay unless values were actually calculated

- recalc deadtime resistors

- check bootstrap parts

- check low-pass filters

fpga / matlab

- matlab is aligned with 3PWM

- schematic names are not aligned rn bc PWM HI / PWM LO makes it look 6PWM

- fpga module should stay 3PWM

- connector pinout should stay 3PWM

- safe reset behavior should stay 3PWM

- hdl coder model should stay 3PWM unless we actually decide to redesign

matlab files

- matlab/foc_fixed_point_starter.m

- Q1.15 fixed point style math

- per-unit scaling

- Q encode / decode

- Q1.15 multiply

- saturation

- anti-windup PI

- Clarke / Park sanity checks

- PWM-center sample timing

- base_current_A = 100

- base_voltage_V = 96

- f_pwm_Hz = 20000

- matlab/stage1_3pwm_foc_sim.m

- stage 1 3PWM FOC signal path

- id / iq PI loops

- inverse Park

- SVPWM-ish duties

- center-aligned PWM

- sample strobe timing

- low voltage bench assumptions first

- base_bus_V = 12

- base_current_A = 20

- f_pwm_Hz = 20000

- assumes fpga outputs PWM_A PWM_B PWM_C only

- gate driver / pcb side handles complementary outputs and deadtime

meeting questions

- final parts? UCC21222D / TMCS1126A1AQDVGRQ1 / BSC110N15NS5

- where exactly are inversion + deadtime made

- should schematic labels change from PWM HI/LO to PWM A/B/C + internal gate-driver names

- are PWM A/B/C, DISABLE, nOC A/B, hall A/B/C, current sense A/B, current refs, bus voltage, temp all going to arty-compatible pins

- are current sensor outputs safe for xadc

- are we reconstructing phase C from 2 sensors

- does nOC go straight to fpga fabric for fast shutdown

- final HV clearances / trace widths / pours / net classes for Vm, phase outputs, switch nodes, isolation

- are GND1 and GND2 rules actually explicit

- where are DC-link caps relative to MOSFET half bridges

- thermal plan for mosfets + current sensors

- what test points / LEDs before sending out

- are placeholder values recalculated

- low voltage bring-up plan before 96 V

pcb next

- rename pwm labels so arty-facing side is clearly 3PWM

- confirm part suffixes / packages / footprints

- replace placeholder R/C values

- add useful test points

- add LEDs if useful

- define net classes for logic / gate drive / high current phase / dc bus / switch node / isolation

- creepage / clearance check

- slots / cutouts maybe?

- gate loop layout review

- current sensor placement + routing review

- DRC after custom rules

fpga / matlab next

- keep fixed point chain in per-unit

- write down Q formats

- overflow checks

- matlab test vectors

- build 3PWM HDL module

- include enable + safe reset + fault kill + sample strobe

- vivado arty z7-10 LED blink

- then pwm output on safe header pins

- AXI lite register map

- decide current data path: xadc / external adc / something else

- hall decoder

- speed estimator

- HDL coder after fixed point stuff makes sense

links

fpga / zynq / vivado / xadc

- Digilent Arty Z7: https://digilent.com/shop/arty-z7-zynq-7000-soc-development-board/

- Digilent Vivado board files: https://github.com/Digilent/vivado-boards

- Zynq-7000 TRM UG585: https://docs.amd.com/r/en-US/ug585-zynq-7000-SoC-TRM

- XADC UG480: https://docs.amd.com/r/en-US/ug480_7Series_XADC

- AXI GPIO PG144: https://docs.amd.com/r/en-US/pg144-axi-gpio

- AXI Interconnect PG059: https://docs.amd.com/r/en-US/pg059-axi-interconnect

- Vivado programming/debugging UG908: https://docs.amd.com/r/2025.1-English/ug908-vivado-programming-debugging

matlab / hdl coder

- HDL Coder FOC PMSM example: https://www.mathworks.com/help/hdlcoder/ug/field-oriented-control-of-a-permanent-magnet-synchronous-machine.html

- Motor Control Blockset FPGA FOC example: https://www.mathworks.com/help/mcb/gs/foc-pmsm-using-mcb-blocks-fpga-hardware.html

- Motor Control Blockset: https://www.mathworks.com/products/motor-control.html

- Fixed-Point Code Generation in Simulink: https://www.mathworks.com/help/fixedpoint/fixed-point-code-generation-in-simulink.html

- Fixed-Point Code Generation Support: https://www.mathworks.com/help/fixedpoint/ug/code-generation-support.html

pcb parts / layout

- UCC21222: https://www.ti.com/product/UCC21222

- TMCS1126: https://www.ti.com/product/TMCS1126

- TMCS1126-Q1: https://www.ti.com/product/TMCS1126-Q1

- BSC110N15NS5: https://www.infineon.com/cms/en/product/power/mosfet/n-channel/bsc110n15ns5/

- TI TIDM-02017 5 kW traction inverter: https://www.ti.com/tool/TIDM-02017

- TI TIDA-010956 48 V 85 Arms inverter: https://www.ti.com/tool/TIDA-010956

- TI TIDA-00909 high frequency inverter: https://www.ti.com/tool/TIDA-00909

simplefoc

- boards: https://docs.simplefoc.com/boards

- code flow: https://docs.simplefoc.com/code

- current sense testing: https://docs.simplefoc.com/test_current

- foc current control: https://docs.simplefoc.com/foc_current

commercial refs

- WaveSculptor22 manual: https://docs.prohelion.com/Motor_Controllers/WaveSculptor22/User_Manual/index.html

- motor sense connections: https://docs.prohelion.com/Motor_Controllers/WaveSculptor22/User_Manual/Motor_Sense_Connections.html

- CAN + LV interface: https://docs.prohelion.com/Motor_Controllers/WaveSculptor22/User_Manual/CAN_Bus_And_Low_Voltage.html

- bring-up checklist: https://docs.prohelion.com/Motor_Controllers/WaveSculptor22/User_Manual/Getting_Started.html

fpga stuff

- fixed point / bit true math

- Id / Iq PI + transforms in matlab until scaling is predictable

- Q formats + saturation points

- test vectors

- 3PWM module

- fault kill before fancy motor control

- enough AXI lite for setpoints + status

- current sampling is pcb + fpga, not just matlab

- HDL coder / vivado after matlab is solid

watch out

- schematic names still look 6PWM

- arty z7-10 I/O limits

- 3.3 V only

- current sensor output range vs xadc

- only 2 current sensors

- phase C reconstruction

- pcb rules still generic

- thermal plan not proven

- safe default-off behavior

- placeholder values

- HV testing needs fusing / precharge / e-stop / low voltage validation

# FOC